## 选型指南

2.3. 电动执行器选型

电动执行器选型需要的参数: 1. 角行程电动执行器： 输出900的转角，控制蝶阀、风门、球阀等需0—900转角打开的阀门。 参数主次; 作用; 参数名称. 主要参数; 选型用; 输出力矩： Nm (牛顿*米). 次要参数; 与控制系统的联系方式; ①输入信号： mA (毫安) ②输出信号： mA (毫安) ③控制方式. 现场机械安装方式; 与阀门的连接方式：直接对接或通过连杆连接. 控制阀门的回转速度; 0—900行程时间： S (秒). 2. 直行程电动执行器： 输出推力、直线位移，控制直通阀等需直线位移打开的阀门。 参数主次; 作用; 参数名称. 主要参数; 选型用; ①行程： mm (毫米) ②推力： N (牛顿). 次要参数; 执行器与控制系统的联系方式; ①输入信号： mA (毫安) ②输出信号： mA (毫安) ③控制方式. 控制阀门的开启速度; 全行程所需时间： S (秒). 3. 多回转电动执行器： 输出超过3600的转角，控制闸板阀等需900以上转角，多圈转动打开的阀门。 参数主次; 作用; 参数名称. 主要参数; 选型用; ①额定力矩： Nm (牛顿*米) ②额定速度： R/min (转/分钟). 次要参数; 执行器与控制系统的联系方式; ①输入信号： mA (毫安) ②输出信号： mA (毫安) ③控制方式. 控制阀门的开启速度; 额定行程： r (转).

## http://www.bellaut.cn/News_83.aspx

## 2.How to Choose Electric Actuator

No; Controlled valves; Equipped electric actuator type. 1; Butterfly valve，damper, ball valve, etc.; Quarter turn type; Output: 0-90 degree. 2; Direct through valve; Linear turn type; Output: linear displacement and push-pull effort. 3; Gate valve; Multi turn type; Output: ≥90 degree. 1. Parameters for choosing quarter turn electric actuator: Output 90 degree, to control butterfly valve, damper and ball valve，which need 0-90 degree to open Parameters; Use; Parameter. Main parameter; For choosing model; Output torque: Nm. Minor parameters; Connecting ways with control system; 1.Input signal: mA 2.Output signal: mA 3.Control way. Mechanical installing ways; Connected ways with valve: connecting directly or through stem. Rotation speed of control valve; 0-90 degree travel time: S (second). 2. Parameters for choosing linear turn electric actuator: Output pulling force, direct linear displacement，control direct through valve, which need to open by direct line displacement Parameters; Use; Parameters. Main Parameters; For choosing model; 1.travel: mm 2.pulling force: N. Minor Parameters; Connecting ways with control system; 1.input signal: mA 2.output signal: mA 3.control way. Opening speed of Control valve; Travel time: S (second). 3. Needed parameter to choose multi turn electric actuator More than 360 degree, to control gate valve and so on, which need multi turn rotation to open Parameters; Use; Parameters. Main Parameters; For choosing model; 1.rated torque: Nm 2.rated speed: turn/minute. Minor Parameters; Connecting ways with control system; 1.input signal: mA 2.output signal: mA 3.control way. Opening speed of Control valve; Rated travel: turn.